Supplementary Data

For

Occurrence and Fate of Quinolone and Fluoroquinolone Antibiotics in a Municipal Sewage Treatment Plant

Ai Jia, Yi Wan, Yang Xiao, and Jianying Hu*

MOE Laboratory for Earth Surface Processes, College of Urban and Environmental Sciences, Peking University, Beijing, 100871 China

Text

Reagents and Materials.

LC-MS/MS Analysis.

Calculations of Mass Flow and Solid-water Partition Coefficient.

Cation Exchange Capacity (CEC) Measurement.

Figures    2

Tables     5

Words     1851

Reagents and Materials. Cinoxacin, lomefloxacin, pipemidic acid, ofloxacin, danofloxacin, enrofloxacin, ciprofloxacin, sarafloxacin, difloxacin, sparfloxacin, moxifloxacin, and fleroxacin were purchased from Sigma (St. Louis, MO, USA); norfloxacin, oxolinic acid, pefloxacin, and flumequine were purchased from Dr. Ehrenstorfen (GmbH, Germany); nalidixic acid was purchased from Acros Organics (Geel, Belgium); piromidic acid was purchased from Wako (Japan); gatifloxacin was purchased from LKT Laboratories Inc. (Minnesota, USA), and norfloxacin-d5 was purchased from RdH Laborchemikalien GmbH (Germany). Chemical structures of all compounds are listed in Figure S1.

Methylene chloride (HPLC grade) was purchased from J.T. Baker, USA, and methanol and acetonitrile (HPLC grade) were obtained from Fisher Chemicals, USA. Formic acid (HPLC grade) was purchased from Dima Technology TNC (USA). Triethylamine, citric acid, monobasic sodium phosphate, dibasic sodium phosphate, hydrochloric acid (HCl), and ethylenediamine tetraacetic acid disodium (Na2EDTA) were of analytical reagent grade and obtained from Sinopharm Chemical Reagent Co., Ltd. (Beijing, China). Oasis HLB (500 mg, 6 mL) cartridge was purchased from Waters Corporation (Milford, MA, USA). Distilled water was purified by a Milli-Q Synthesis water purification system (Millipore, Bedford, MA, USA).

LC-MS/MS Analysis. Analyses were carried out using a Waters Acquity UPLCTM system (Waters, Milford, MA, USA). Separation of target antibiotics was achieved with a Waters Acquity UPLC BEH C18 column (100 mm × 2.1 mm, 1.7 µm). The column was maintained at 40°C at a flow rate of 0.2 mL/min, and the injection volume was 2 μL. Methanol (A) and purified water containing 0.1% formic acid (v/v) (B) were used as mobile phases. The gradient was as follows: initial 10% A was increased linearly to 30% in 6 min and then to 50% in 3 min. Then A was increased to 100% in 0.5 min and maintained for 3 min. At 0.1 min, A was then sharply returned to the initial percentage and held for 4 min to allow for equilibration.

Mass spectrometry was performed using a Waters Micromass Quattro Premier XE detector equipped with an electrospray ionization source (Micromass, Manchester, UK). The MS parameters were as follows: capillary voltage, 3.6 kV; source temperature, 110°C; desolvation temperature, 350°C; source gas flow, 50 L/h; and desolvation gas flow, 800 L/h. Quantitative analysis was performed in the selected reaction monitoring (MRM) mode, and time-segmented scanning was used to obtain maximum sensitivity of the ESI+ mode. The precursor and product ions, cone voltage, and collision energy of all analytes are summarized in Table S1.

Calculations of Mass Flow and Solid-water Partition Coefficient. Mass flow was calculated by multiplying the measured concentration of each detected compound by the respective water flow or solid amount during the sampling period.  Mass balances were performed using the sum of mass flow in dissolved and sorbed phases:

(1)

where  is the total mass of individual compound in aqueous and sorbed phase;  and  represent the dissolved and sorbed concentrations, respectively;  is the water flow; and  represents the concentration of total suspended solids. Detailed STP parameters are listed in Table S3.

The sorption behavior of FQs was assessed using the apparent solid-water partition coefficient (Kd, L/kg) as calculated by , where  and  represent the chemical concentrations in sorbed and dissolved phase from the activated sludge treatment units, respectively.

Cation Exchange Capacity (CEC) Measurement. CEC values in sludge samples collected from aerobic, anoxic and anaerobic units were measured assess the variations of Kd values in different treatment units.  The freeze-dried sludge samples were grounded and sieved through a 0.2 mm mesh.  About 0.5 g sludge was weighed into a 50 mL centrifuge tube, and immediately added with 20 mL 1 M ammonium acetate (NH4OAc, pH=7.0).  The tubes were shake in a rotor shaker for two hours, and centrifuged for 10 min at 6,000 g.  15 mL supernatant was diluted with 10 times, and analyzed for exchangeable Ca, Mg, Na and K by Inductively Coupled Plasma (ICP)-Atomic Emission Spectrometer (AES).  And acidity in the sludge was measured by titration with 0.01 M NaOH solution.  CEC was estimated by summation of exchangeable bases (Ca, Mg, Na and K) and exchangeable acidity.

| 1st |  |
| --- | --- |
| 2nd |  |
| 3rd |  |
| 4th |  |

Figure S1 Structures of the target quinolone and fluoroquinolone antibiotics. The red and green shades were the locations of the negatively and positively-charged moieties for each compound.  Positively-charged moieties for each chemical should be occurred at nitrogen atoms which interact with H ion. Of nitrogen atoms in a chemicals, the nitrogen atom with the lowest atom partial charge was most easily interact with H ion, and therefore a potential positively-charged moiety.  The atom partial charges were calculated by MOPAC (version 6, CAChe Scientific Inc., Oxford, U.K.), and the PM3 parameter was served to optimize stable structures (Hu et al., 2002).

Figure S2. Schematic diagram of the treatment processes in Qinghe STP.

Table S1. Multi-selected Reaction Monitoring (MRM) Conditions of the Target Quinolone and Fluoroquinolone Antibiotics.

| Function (min) | Compound | MRM transition | Cone voltage (V) | Collision energy (eV) | Function (min) | Compound | MRM transition | Cone voltage (V) | Collision energy (eV) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4-7.1 | pipemidic acid | 304>217 304>286 | 31 | 22 16 | 6.7-7.8 | lomefloxacin | 352>265 352>308 | 34 | 22 16 |
|  | flerofloxacin | 370>269 370>326 | 37 | 25 19 |  | difloxacin | 400>299 400>356 | 45 | 28 20 |
|  | ofloxacin | 362>261 362>318 | 34 | 25 19 |  | sarafloxacin | 386>299 386>342 | 45 | 28 18 |
|  | pefloxacin | 334>290 334>316 | 37 | 19 19 | 7.8-9.4 | gatifloxacin | 376>289 376>332 | 37 | 28 19 |
|  | norfloxacin | 320>276 320>302 | 31 | 16 19 |  | sparfloxacin | 393>292 393>349 | 40 | 24 20 |
|  | norfloxacin-d5 | 325>281 325>307 | 31 | 19 22 |  | moxifloxacin | 402>358 402>384 | 34 | 19 22 |
|  | ciprofloxacin | 332>288 332>314 | 37 | 34 16 |  | cinoxacin | 263>189 263>245 | 30 | 28 15 |
|  | danofloxacin | 358>283 358>340 | 40 | 25 25 | 9.2-11.5 | oxolinic acid | 262>216 262>244 | 32 | 25 18 |
|  | enrofloxacin | 360>316 360>342 | 42 | 20 22 |  | nalidixic acid | 233>187 233>215 | 30 | 25 14 |
|  |  |  |  |  |  | flumequine | 262>202 262>244 | 28 | 34 22 |
|  |  |  |  |  |  | piromidic acid | 289>243 289>271 | 32 | 30 18 |

Table S2. Efficiency of Different Extraction Solutions to Target Quinolone and Fluoroquinolone Antibiotics from the Sludge Samples, the Optimized Method Detection Limits (MDLs), and Matrix Effects.

| Chemicals | Solution A×3 | Solution B×3 | Solution C×1 + Solution D×1 | Solution C×1 + Solution D×2 | Solution D×3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Recovery (%) | Recovery (%) | Recovery (%) | Recovery (%) | Recovery (%) | Matrix effect (%） | MDL (mg/Kg) |
| pipemidic acid | 11±65 | 16±83 | 55±3.0 | 44±20 | 78±1.4 | -1.4±2.8 | 0.014 |
| fleroxacin | 56±31 | 15±114 | 97±2.5 | 91±2.6 | 82±21 | -1.7±3.5 | 0.0085 |
| ofloxacin | 71±33 | 17±94 | 80±3.1 | 83±7.4 | 87±1.3 | 25±3.1 | 0.028 |
| pefloxacin | 42±4 | 19±73 | 55±6.0 | 60±11 | 68±3.4 | -11±1.0 | 0.085 |
| norfloxacin | 40±43 | 20±102 | 61±0.5 | 62±10 | 74±4.1 | 26±12 | 0.036 |
| norfloxacin-d5 | 21±18 | 15±18 | 49±9.8 | 59±13 | 87±2.4 | 17±6.0 | 0.026 |
| ciprofloxacin | 23±28 | 27±63 | 62±2.7 | 60±13 | 74±2.4 | -13±4.1 | 0.028 |
| danofloxacin | 57±17 | 31±76 | 85±15 | 77±0.5 | 77±9.8 | -12±1.2 | 0.069 |
| enrofloxacin | 27±30 | 52±48 | 62±5.0 | 76±4.7 | 86±9.8 | -12±1.5 | 0.0094 |
| lomefloxacin | 78±13 | 29±76 | 84±5.4 | 91±2.1 | 88±0.7 | -6.4±0.1 | 0.043 |
| difloxacin | 69±9.9 | 54±52 | 84±6.9 | 84±8.7 | 81±15 | -2.5±0.9 | 0.020 |
| sarafloxacin | 37±11 | 67±39 | 73±2.2 | 85±4.5 | 74±1.6 | 3.9±0.95 | 0.039 |
| gatifloxacin | 80±7.2 | 61±62 | 83±1.9 | 84±1.7 | 80±0.9 | 2.8±4.3 | 0.021 |
| sparfloxacin | 77±1.9 | 65±54 | 78±1.5 | 78±4.7 | 73±4.0 | 12±1.0 | 0.0061 |
| moxifloxacin | 70±6.8 | 70±64 | 90±4.8 | 81±2.2 | 77±6.6 | -4.5±3.9 | 0.032 |
| cinoxacin | 87±9.4 | 94±22 | 94±1.9 | 75±3.3 | 74±7.1 | -19±4.3 | 0.041 |
| oxolinic acid | 67±6.5 | 89±33 | 81±1.1 | 92±13 | 78±23 | 20±2.2 | 0.031 |
| nalidixic acid | 90±7.3 | 90±22 | 91±0.4 | 67±2.2 | 67±4.7 | -0.7±2.8 | 0.015 |
| flumequine | 97±3.5 | 95±14 | 102±2.5 | 78±5.3 | 78±3.6 | -8.9±8.0 | 0.020 |
| piromidic acid | 53±15 | 81±11 | 66±8.3 | 84±1.7 | 98±4.3 | 1.7±2.6 | 0.027 |

Solution A: methanol/citrate buffer (1/1, pH 2.6); solution B: 1/1 acetonitrile/phosphate buffer (pH 2.2); Solution C: phosphate buffer (pH 6.0); solution D: 5/25/75 triethylamine/methanol/purified water (pH 11.0).

Table S3. Treatment Parameters in Qinghe STP during the Sampling Period.

|  |  | raw sewage | primary effluent | primary sludge | anaerobic | anoxic | aerobic | internal recirculation | secondary effluent | return sludge | excess sludge | Supernatant of excess sludge | dewatered sludge |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| water flow (m3/d) |  | 2×105 | 2×105 | - | 4×105 | 10×105 | 10×105 | 6×105 | 2×105 | 1.90×105 | 0.023×105 | 0.028×105 | - |
|  |  |  |  |  |  |  |  |  |  | 1.75×105 |  | 0.025×105 |  |
|  |  |  |  |  |  |  |  |  |  | 2.28×105 |  | 0.013×105 |  |
| TSS (Kg/d) | 7/7 | 6.16×105 | 3.70×105 | - | 17.5×105 | 43.8×105 | 43.8×105 | 26.3×105 | 1.23×104 | 17.6×105 | 2.31×105 | - | 0.29×105 |
|  | 7/8 | 6.24×105 | 3.74×105 |  | 17.1×105 | 42.8×105 | 42.8×105 | 25.7×105 | 1.25×104 | 16.8×105 |  |  | 0.26×105 |
|  | 7/11 | 4.48×105 | 2.69×105 |  | 19.1×105 | 47.8×105 | 47.8×105 | 28.7×105 | 0.90×104 | 17.9×105 |  |  | 0.14×105 |
| COD (mg/L) |  | 476 |  |  |  |  |  |  | 41.3 |  |  |  |  |
| BOD5 (mg/L) |  | 229 |  |  |  |  |  |  | 9.06 |  |  |  |  |
| DO (mg/L) |  |  |  |  |  |  |  |  | 3.52 |  |  |  |  |
| NH4-N (mg/L) |  |  |  |  |  |  |  |  | 0.672 |  |  |  |  |
| NO3-N (mg/L) |  |  |  |  |  |  |  |  | 10.8 |  |  |  |  |
| TP (mg/L) |  | 5.89 |  |  |  |  |  |  | 0.214 |  |  |  |  |
| pH |  |  |  |  |  |  |  |  | 7.74 |  |  |  |  |
| Temp (0C) |  |  |  |  |  |  |  |  | 24.4 |  |  |  |  |

Table S4. Mass flux (g/d) of Detected Quinolone and Fluoroquinolone Antibiotics in Qinghe STP.

| Chemicals | Raw sewage |  | Primary effluent | primary sludge | anaerobic |  | anoxic |  | aerobic |  | internal recirculation |  | secondary effluent | return sludge |  | excess sludge |  | supernatant of excess sludge | dewatered sludge |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dissolved | sorbed |  |  | dissolved | sorbed | dissolved | sorbed | dissolved | sorbed | dissolved | sorbed |  | dissolved | sorbed | dissolved | sorbed |  |  |
| pipemidic acid | 17±3.4 | 1.2±0.8 | 16±2.5 | 1.6±2.0 | 11±1.4 | 511±21 | 22±6.3 | 1226±25 | 22±7.9 | 1329±163 | 13±4.7 | 798±98 | 6.5±2.4 | 3.7±2.2 | 553±160 | 0.12±0.04 | 6.2±1.5 | 0.08±0.01 | 6.2±1.5 |
| fleroxacin | 2.7±0.3 | 1.0±0.3 | 2.9±0.26 | 0.74±0.15 | 2.9±0.26 | 136±10 | 4.9±0.44 | 363±13 | 5.2±0.95 | 430±45 | 3.1±0.57 | 258±27 | 1.0±0.07 | 0.72± 0.19 | 149±21 | 0.02±0.01 | 1.9±0.1 | 0.02±0.01 | 2.1±0.5 |
| ofloxacin | 257±19.4 | 19±2.3 | 315±20.2 | 11±0.4 | 219±66.0 | 11417± 1135 | 384±21.5 | 32550± 4828 | 336±70.4 | 32804± 2371 | 202±42.2 | 19683± 1423 | 106±17.8 | 48±23.9 | 13174±754 | 2.6±1.0 | 171±69 | 1.6±0.31 | 180±55 |
| norfloxacin | 155±17.5 | 46±14 | 166±19.5 | 34±5.1 | 100±11.5 | 11482± 1284 | 207±38.1 | 29847± 1157 | 190±52.4 | 33715± 1239 | 114±31.4 | 20229± 744 | 51±12.9 | 32±22.2 | 12594± 1439 | 1.4±0.5 | 148±49 | 0.78±0.08 | 167±61 |
| ciprofloxacin | 20±4.3 | 11±1.9 | 21±2.6 | 7.4±3.7 | 15±1.9 | 1670±143 | 30±4.9 | 4434±93 | 27±7.2 | 4687±319 | 16±4.3 | 2812±191 | 7.4±3.2 | 4.8±3.2 | 2000±168 | 0.20±0.08 | 22±7.1 | 0.11±0.02 | 24±11 |
| enrofloxacin | 1.7±0.6 | 1.0±0.1 | 1.3±0.63 | 0.54±0.17 | 1.0±0.43 | 171±10 | 2.5±0.67 | 349±11 | 2.6±1.11 | 401±163 | 1.5±0.66 | 241±98 | 0.41±0.13 | 0.15±0.11 | 171±22 | 0.01±0.01 | 1.7±0.3 | 0.01±0.01 | 1.7±0.6 |
| lomefloxacin | 32±0.7 | 3.2±0.0 | 37±5.1 | 2.0±0.5 | 34±8.6 | 1807±112 | 71±16.3 | 4365±259 | 61±17.2 | 4732±291 | 37±10.3 | 2839±175 | 14±2.7 | 11±6.4 | 2044±324 | 0.67±0.24 | 23±8.3 | 0.25±0.01 | 23±9.1 |
| sarafloxacin | - | - | - | - | - | 1025±66 | - | 2191±53 | - | 2273±217 | - | 1364±130 | - | - | 1075±199 | - | 13±4.1 | - | 12±4.1 |
| gatifloxacin | 13±1.3 | 3.6±0.6 | 14±1.8 | 3.0±0.7 | 15±4.2 | 693±90 | 28±3.5 | 1809±70 | 25±5.1 | 2136±38 | 15±3.1 | 1282±23 | 7.9±1.6 | 4.0±1.7 | 824±174 | 0.25±0.09 | 10±4.0 | 0.10±0.01 | 11±4.4 |
| sparfloxacin | 0.9±0.06 | 0.5±0.1 | 0.78±0.07 | 0.4±0.2 | 0.39±0.09 | 55±4.9 | 0.72±0.04 | 113±7.7 | 0.74±0.17 | 151±32 | 0.44±0.10 | 91±19 | 0.22±0.03 | 0.12± 0.08 | 49±7.8 | 0.01±0.01 | 0.78±0.31 | 0.003±0.001 | 0.86±0.24 |
| moxifloxacin | 14±6.8 | 9.1±0.3 | 20±8.9 | 5.7±0.4 | 14±1.0 | 908±86 | 31±4.2 | 2042±52 | 33±7.1 | 2499±142 | 20±4.3 | 1499±85 | 8.0±4.1 | 4.2±2.9 | 989±202 | 0.19±0.08 | 13±4.6 | 0.10±0.01 | 12±6.3 |

Table S5. Calculated Coefficients and Solid-water Partition Coefficients (Kd, L/Kg) of Detected Quinolone and Fluoroquinolone Antibiotics.

| Chemicals | pKaa | logPow (at pH7.74)b | Kd |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Anaerobic | Anoxic | Aerobic | Average |
| sparfloxacin | 8.88 | 4.53 | 31900 | 35300 | 45800 | 37700 |
| enrofloxacin | 7.11 | 1.80 | 39000 | 32700 | 35800 | 35800 |
| norfloxacin | 8.76 | 1.45 | 25700 | 32500 | 40500 | 32900 |
| pipemidic acid | 8.75 | 1.02 | 10300 | 12500 | 14100 | 12300 |
| ciprofloxacin | 8.76 | 1.29 | 25400 | 33000 | 38900 | 32400 |
| fleroxacin | 7.03 | 1.54 | 10600 | 16500 | 18500 | 15200 |
| ofloxacin | 6.81 | -0.02 | 12000 | 18800 | 22100 | 17600 |
| lomefloxacin | 8.80 | 2.31 | 12100 | 14000 | 17600 | 14600 |
| moxifloxacin | 10.68 | 2.49 | 14000 | 14900 | 17000 | 15300 |
| gatifloxacin | 8.82 | 2.27 | 10800 | 14300 | 19100 | 14700 |

a, ref (1); b, logPow was calculated by ACD/logPow ver. 1.0 (Advanced Chemistry Development, Inc.), and the average pH values in raw sewage and secondary effluent were all 7.74 during the sampling period in Qinghe STP.

REFERENCE

Hu, J.Y., Wang, W.F., Zhu, Z., Chang, H., Pan, F., Lin, B.L., 2007. Quantitative structure - activity relationship model for prediction of genotoxic potential for quinolone antibacterials. Environ. Sci. Technol. 41, 4806-4812.

Hu, J. Y., Aizawa, T., Ookubo, S. 2002. Products of aqueous chlorination of bisphenol A and their estrogenic activities. Environ. Sci. Technol. 36, 1980-1987.